Total Consumption (KTA/ MGPY/ BPD)

Input: KTA

Output: feedstock/ H2/ water (KTA)

Consumption = Plant total liquid fuel production capacity X feedstock yield

Output: Electricity (kWh/year)

Consumption = Plant total liquid fuel production capacity X 1E6 X feedstock yield

Input: MGPY

Output: feedstock/ H2/ water (KTA)

Consumption = Plant total liquid fuel production capacity X 3785.4118 (m3/MG) X Average Density (kg/m3) X feedstock yield / 1E6 (kg/KT)

Output: Electricity (kWh/year)

Consumption = Plant total liquid fuel production capacity X 3785.4118 (m3/MG) X Average Density (kg/m3) X feedstock yield

Convert million gallons to cubic metre - Conversion of Measurement Units

Input: BPD

Output: feedstock/ H2/ water (KTA)

Consumption = Plant total liquid fuel production capacity X 0.1589872949 (m3/barrel) X Average Density (kg/m3) X (plant annual load hours/ 24) X feedstock yield / 1E6 (kg/KT)

Output: Electricity (kWh/year)

Consumption = Plant total liquid fuel production capacity X 0.1589872949 (m3/barrel) X Average Density (kg/m3) X (plant annual load hours/ 24) X feedstock yield

Convert Barrel (oil) to Cubic Meter

*Assuming plant runs 24 hrs per day

Production (KTA)

Input: KTA

Output: Each Fuel (KTA)

Production (jet) = Plant total liquid fuel production capacity X product yield jet

Production (Diesel) = Plant total liquid fuel production capacity X product yield diesel

Production (Naphtha) = Plant total liquid fuel production capacity X product yield naphtha

Input: MGPY

Output: Each Fuel (KTA)

Production (jet) = Plant total liquid fuel production capacity (MGPY) X 3785.4118 (m3/MG) X Density jet (kg/m3) X product yield jet / 1E6 (kg/KT)

Production (Diesel) = Plant total liquid fuel production capacity (MGPY) X 3785.4118 (m3/MG) X Density diesel (kg/m3) X product yield diesel / 1E6 (kg/KT)

Production (Naphtha) = Plant total liquid fuel production capacity (MGPY) X 3785.4118 (m3/MG) X Density naphtha (kg/m3) X product yield naphtha / 1E6 (kg/KT)

Convert million gallons to cubic metre - Conversion of Measurement Units

Input: BPD

Output: Each Fuel (KTA)

Production (jet) = Plant total liquid fuel production capacity (BPD) X 0.1589872949 (m3/barrel) X Density jet (kg/m3) X product yield jet / 1E6 (kg/KT) X (plant annual load hours/ 24)

Production (Diesel) = Plant total liquid fuel production capacity (BPD) X 0.1589872949 (m3/barrel) X Density diesel (kg/m3) X product yield diesel / 1E6 (kg/KT) X (plant annual load hours/ 24)

Production (Naphtha) = Plant total liquid fuel production capacity (BPD) X 0.1589872949 (m3/barrel) X Density naphtha (kg/m3) X product yield naphtha / 1E6 (kg/KT) X (plant annual load hours/ 24)

Convert Barrel (oil) to Cubic Meter

*Assuming plant runs 24 hrs per day

Carbon Intensity (kg CO2e/ton P)

HEFA

EC product = (EC jet*Yield jet)+(EC diesel*Yield diesel)+(EC Naphtha*Yield Naphtha)

Carbon intensity total (per unit product) = [carbon intensity (feedstock + hydrogen + electricity) + carbon intensity (process)] * EC product

CI feedstock UCO/hydrogen= feedstock CI gCO2ekg feedstock×feedstock yield kg feedstockkg fuelfuel energy content MJkg fuel

CI electricity= feedstock CI gCO2ekWh×feedstock yield kWhkg fuelfuel energy content MJkg fuel

CI process=CI conversion process (gCO2eMJ)

Each Fuel

CI (jet) = CI total X product yield jet

CI (Diesel) = CI total X product yield diesel

CI (Naphtha) = CI total X product yield naphtha

PTL-FT-DAC

Carbon intensity (per unit product) = [carbon intensity (electricity) + carbon intensity (process)]/1000

CI electricity= feedstock CI gCO2ekWh×feedstock yield kWhkg fuelfuel energy content MJkg fuel

CI process=CI conversion process (gCO2eMJ)

Each Fuel

CI (jet) = CI total X product yield jet

CI (Diesel) = CI total X product yield diesel

CI (Naphtha) = CI total X product yield naphtha

Carbon Conversion Efficiency (%C converted)

Each Fuel

CCE (jet) = (CC jet X jet yield / CC feedstock X feedstock yield) X 100%

CCE (Diesel) = (CC diesel X diesel yield/ CC feedstock X feedstock yield) X 100%

CCE (Naphtha) = (CC naphtha X naphtha yield / CC feedstock X feedstock yield) X 100%

*If multiple feedstocks are used, CC feedstock should the sum of all feedstocks carbon content

Total CO2 emissions (ton CO2e/yr)

Each Fuel

Total CO2 emission (jet) = CI (jet) X production jet / 1000

Total CO2 emission (Diesel) = CI (diesel) X production diesel / 1000

Total CO2 emission (Naphtha) = CI (naphtha) X production naphtha / 1000